Lipid Extraction from Tissues Using the Matyash Method

## 1.	Purpose

The purpose of this SOP is to outline the standardized procedure for extracting lipids from tissue for targeted and untargeted lipidomics analysis by LC-MS. The procedure is designed to maximize recovery of diverse lipid classes while maintaining reproducibility and comparability across experiments.

## 2.	Scope

This SOP outlines the procedure for extracting lipids from tissue samples using a biphasic MTBE/methanol/water solvent system based on Matyash, V., et al. (2008) for sphingolipid profiling in murine tissues. The protocol supports targeted and untargeted lipidomics workflows and is compatible with high-throughput batch analysis.

## 3.	Responsibility

- Laboratory staff must follow this SOP as written.

- The lab manager or designated supervisor is responsible for training, compliance, and periodic review of this procedure.

## 4.	Safety Considerations

- Perform all steps in a fume hood.

- Wear appropriate PPE: lab coat, gloves, and safety glasses.

- MTBE, methanol, acetonitrile, and isopropanol are flammable and toxic—consult safety data sheets (SDS) prior to use.

- Dispose of organic solvents and biological waste according to institutional regulations.

## 5.	Materials and Reagents

| Item | Details |
| --- | --- |
| Phosphate Buffered Saline (PBS) | LC-MS grade (e.g. Fisher 10010031) |
| Methanol (MeOH) | LC-MS grade (e.g. Honeywell B&J LC-230-4) |
| Water | LC-MS grade (18.2 MΩ·cm or equivalent) |
| Methyl-tert-butyl ether (MTBE) | LC-MS grade (e.g. Fisher E127-4) |
| Isopropanol (IPA) | LC-MS grade |
| Tissue homogenizer | Bead mill or rotor-stator (e.g., Omni Bead Ruptor Elite) |
| Internal standards | See Section 7.1 |
| Tubes | Solvent-resistant, glass or polypropylene, 1.5–5 mL |
| Centrifuge | Capable of ≥14,000 × g |
| Vortexer and sonicator | For mixing and solubilizing |
| Nitrogen evaporator/SpeedVac | For drying pellets |
| LC-MS vials | e.g., Agilent 5182-0554 |

## 6.	Sample Requirements

- Recommended starting material: 5–30 mg frozen tissue

- Accurately record wet weight before extraction

- Dry weight may be determined by post-extraction for normalization

## 7.	Reagent Preparation

7.1 Extraction Solvent with Internal Standards

Add internal standards below directly to the methanol, ensuring the final volume of methanol added to each sample is 225 µL:

| Standard | Avanti PN | pmol/sample |
| --- | --- | --- |
| C16 Ceramide-d7 (d18:1-d7/16:0) | 860676P-1mg | 367.0 |
| C18 Ceramide-d7 (d18:1-d7/18:0) | 860677P-1mg | 349.0 |
| C24 Ceramide-d7 (d18:1-d7/24:0) | 860678P-1mg | 304.3 |
| C24:1 Ceramide-d7 (d18:1-d7/24:1) | 860679P-1mg | 305.3 |
| C8 Dihydroceramide (d18:0/8:0) | 860626 5 mg | 77.9 |
| N-12:0-1-deoxysphinganine | 860481 1mg | 35.6 |
| C17 Glucosylceramide (d18:1/17:0) | 860569P-5mg | 280.1 |
| sphinganine-1-phosphate-d7 | 860694P-1mg | 34.3 |
| sphingosine-1-phosphate-d7 | 860659P-1mg | 34.5 |
| Sphingosine-d7 | 860657P-1mg | 32.6 |
| SM(d18:1/16:0-d31) | 868584 | 681.0 |
| PC(15:0-18:1(d7)) | 791637C-1mg | 663.9 |
| DG(15:0-18:1(d7)) | 791647C-1mg | 850.4 |
| TG(15:0-18:1(d7)-15:0) | 791648C-1mg | 615.5 |

- Store solvent at −20 °C if not used immediately. Bring to room temperature before use.

-	Suggested standards and amounts shown here, consult the core when needed.

## 8.	Procedure

8.1 Homogenization

- Record the weight of the empty tube.

- Weigh 5–30 mg frozen tissue and record the wet weight.

- Add 188 µL of PBS and 225 µL of MeOH containing internal standards to bead mill tubes containing tissue (from Section 7.1).

- Homogenize samples using a bead mill homogenizer (e.g., Omni Bead Ruptor Elite) for 15 seconds.

- Inspect each sample to confirm complete homogenization into the solvent if particulate clumps or intact material remain, repeat bead mill homogenization for an additional 15 seconds.

8.2 Extraction and Centrifugation

- Transfer homogenates to fresh 1.5 mL Eppendorf tubes containing 750 µL of MTBE.

- Incubate on ice for 60 minutes with intermittent vortexing.

- Centrifuge samples at 14,000 × g for 10 minutes at 4 °C.

- Collect the upper organic phase and dry under nitrogen or speedvac.

8.3 Re-extraction, Centrifugation, and Re-Suspension

- Add 645 µL MTBE, 194 µL methanol, and 161 µL LC-MS-grade water (10:3:2.5, v/v/v) directly to the residual aqueous phase from the first extraction (per sample).

- Incubate on ice for 15 minutes.

- Centrifuge again at 14,000 × g for 10 minutes at 4 °C.

- Collect only the upper organic phase (MTBE-rich layer) without disturbing the interphase or aqueous layer and combine with first extract.

- Evaporate combined organic phases to dryness.

- Reconstitute the dried lipid extract in IPA.

- Centrifuge the reconstituted extracts (14,000 × g, 10 minutes, 4 °C).

- Transfer the clear supernatant to LC-MS vials for analysis.

## 9.	Pooled Technical Quality Control (QC)

- Prepare QC samples by pooling equal volumes (e.g., 25 µL) from each extract.

## 10.	Process Blank (Extraction Blank)

- Prepare one blank per batch using extraction solvent in an empty tube.

- Process identically to samples (vortex, sonicate, centrifuge, dry if applicable).

## 11.	Documentation

- Record:

  - Sample ID, tissue type, wet weight, dry weight (if used)

  - Extraction and homogenization dates

  - Any deviations or sample issues

  - Reagents and lot numbers

## 12.	Notes

- Randomize extraction order.

- Short-term storage 4 to -20 °C; long term storage -20 to -80 °C.

- If particulates are observed or disrupted after centrifugation, then re-centrifuge sample.

- Produce double blank for each batch; constructed by just using re-suspension buffer.

## 13.	References

- Matyash, V., et al. (2008). Lipid extraction by methyl-tert-butyl ether for high-throughput lipidomics. J. Lipid Res. 49(5):1137–1146. https://doi.org/10.1194/jlr.D700041-JLR200