Standard Operating Procedure (SOP)
Title: Lipid Extraction from Cultured Cells Using the Modified Matyash Method
Version: 1.1
Effective Date: July 18, 2025
Prepared by: Trevor Lonergan
Reviewed by: Alan Maschek
Applicable Instrumentation: LC-MS lipidomics

1. Scope

This SOP outlines the procedure for extracting lipids from cultured cells using a biphasic MTBE/methanol/water solvent system based on Matyash et al. (2008). This method is optimized for targeted and untargeted lipidomics workflows.

2. Safety Considerations

- Perform all extractions in a certified chemical fume hood.

- Use appropriate PPE: lab coat, safety glasses, chemical-resistant gloves.

- MTBE, methanol, acetonitrile, and isopropanol are flammable and toxic—consult SDS before use.

- Dispose of organic solvents and biological waste according to institutional and regulatory guidelines.

3. Materials and Reagents

| Item | Details |
| --- | --- |
| PBS | LC-MS grade (e.g., Fisher 10010031) |
| Methanol | LC-MS grade (e.g., Honeywell B&J LC-230-4) |
| MTBE | LC-MS grade |
| Water | LC-MS grade (18.2 MΩ·cm or higher) |
| Homogenizer | Bead mill or rotor-stator (e.g., Omni Bead Ruptor Elite) |
| Internal standards | See Section 5.1 |
| Tubes | Solvent-resistant, glass or polypropylene, 1.5–5 mL |
| Centrifuge | Capable of ≥14,000 × g |
| Vortexer and sonicator | For mixing and solubilization |
| Nitrogen evaporator / SpeedVac | For drying lipid extracts |
| LC-MS vials | e.g., Agilent 5182-0554 |

4. Sample Requirements

- Recommended input: 0.5–5 million cultured cells

- Record the total viable cell count prior to extraction

- Optional: Perform BCA protein assay for normalization

5. Reagent Preparation

5.1 Extraction Solvent with Internal Standards

Prepare methanol/MTBE mixture and spike with internal standards before use.

| Standard | Avanti PN | pmol/sample |
| --- | --- | --- |
| C16 Ceramide-d7 (d18:1-d7/16:0) | 860676P-1mg | 367.0 |
| C18 Ceramide-d7 (d18:1-d7/18:0) | 860677P-1mg | 349.0 |
| C24 Ceramide-d7 (d18:1-d7/24:0) | 860678P-1mg | 304.3 |
| C24:1 Ceramide-d7 (d18:1-d7/24:1) | 860679P-1mg | 305.3 |
| C18:1 Dihydroceramide | 860624P-5mg | 44.2 |
| C8 Dihydroceramide | 860626-5mg | 77.9 |
| N-12:0-1-deoxysphinganine | 860481-1mg | 35.6 |
| C17 Glucosylceramide | 860569P-5mg | 280.1 |
| Sphinganine-1-phosphate-d7 | 860694P-1mg | 34.3 |
| Sphingosine-1-phosphate-d7 | 860659P-1mg | 34.5 |
| Sphingosine-d7 | 860657P-1mg | 32.6 |
| 16:0-d31 SM | 868584 | 681.0 |
| PC(15:0-18:1(d7)) | 791637C-1mg | 663.9 |
| DG(15:0-18:1(d7)) | 791647C-1mg | 850.4 |
| TG(15:0-18:1(d7)-15:0) | 791648C-1mg | 615.5 |

- Store prepared solvent at −20 °C.

- Bring to room temperature before use.

- Consult lipidomics core for standard updates.

6. Procedure

6.1 Cell Resuspension and Protein Aliquot (if applicable)

- Label microcentrifuge tubes and record sample ID.

- Resuspend cell pellet in 208 µL cold water. 188 µL if not taking aliquot for BCA.

- Vortex thoroughly. If clumps remain, probe-sonicate 5–10 sec on ice.

- Optional – BCA protein assay:

  - Remove 20 µL of the PBS suspension to a clean tube.

  - Store at −80 °C until assay.

6.2 Extraction and First Centrifugation

- Add 975 µL MTBE/MeOH (10:3, v/v) containing internal standards.

- Vortex briefly and incubate on ice for 60 minutes, vortexing every 15 minutes.

- Centrifuge at 14,000 × g, 10 minutes, 4 °C.

- Carefully transfer the upper organic phase (MTBE-rich) to a new glass tube using a glass Pasteur pipette, avoiding disturbance of the interphase

- Dry under nitrogen stream or SpeedVac (≤40 °C).

6.3 Re-extraction, Pooling, and Resuspension

- To the remaining lower phase, add 1 mL of MTBE:methanol:water (10:3:2.5, v/v/v).

- Vortex for 30 seconds and incubate on ice for 15 minutes.

- Centrifuge again at 14,000 × g for 10 minutes at 4 °C.

- Collect the upper organic phase and combine with the first extract.

- Evaporate combined extract to dryness under nitrogen or SpeedVac (≤40 °C).

- Resuspend dried lipids in 500 µL isopropanol per 1 million cells, then vortex 10-15 seconds to ensure complete resuspension.

- Centrifuge resuspended samples at 14,000 × g for 10 minutes at 4 °C.

- Transfer supernatant to labeled LC-MS vials. Use glass inserts for volumes < 400 µL.

7. Quality Control

- Prepare pooled technical QC samples by combining equal aliquots (e.g., 10 µL) from each extract.

8. Process Control Samples

8.1 Process Blank (Extraction Blank)

- Add 188 µL PBS to an empty tube and process identically to samples.

- Use to monitor for contamination introduced during extraction and handling.

8.2 Double Blank (Solvent Blank)

- Transfer 500 µL resuspension solvent directly from the sample tube (e.g., Eppendorf) into an LC–MS vial, with no processing. Use to assess background from solvent and vialware.

9. Documentation

Record the following for each sample:

- Sample ID and cell type. Cell count prior to extraction. Reagents and lot numbers used. Dates and personnel involved. Deviations or issues encountered during processing

10. Notes

- Process all samples under consistent and cold conditions. Avoid repeated freeze-thaw of extracts. Re-centrifuge extracts if particulates are observed post-resuspension. Use consistent timing for incubations across batches. Store dried lipid extracts at −20 °C (short term) or −80 °C (long term).

11. References

Matyash, V., et al. (2008). Lipid extraction by methyl-tert-butyl ether for high-throughput lipidomics. J. Lipid Res., 49(5):1137–1146. https://doi.org/10.1194/jlr.D700041-JLR200